A

Project Work-II Report

on

Automatic Solar Panel Cleaning Robot

Submitted in Partial Fulfilment of the Academic Requirements of Degree

Bachelor of Engineering

in

ELECTRONICS AND COMMUNICATION ENGINEERING

by

M. Sri Charan Siddharth                2451-21-735-178

Gouni Sandeep                                2451-21-735-315

Tukkapuram Rohit Chary               2451-21-735-317

Under the Guidance of

Dr. B. Sarala

Professor, ECED

Department of Electronics and Communication Engineering

## Maturi Venkata Subba Rao (MVSR) Engineering College

## An UGC Autonomous Institute Nadergul P.O., Hyderabad - 501510

April 2025

CERTIFICATE

This is to certify that the Project work “Automatic Solar Panel Cleaning Robot”, being submitted by, M. Sri Charan Siddharth, Gouni Sandeep and Tukkapuram Rohit Chary, in partial fulfilment for the award of Bachelor of Engineering (BE) degree, with specialization Electronics and Communication Engineering (ECE), to the Department of Electronics and Communication Engineering, MATURI VENKATA SUBBA RAO (MVSR) ENGINEERING COLLEGE, an autonomous institution under OSMANIA UNIVERSITY, Hyderabad, is a record of the bonafide work carried out by him/her under my guidance and supervision.

Signature of the Guide

Dr. B. Sarala

Professor, ECED

Head of the Department

Dr. G. Kanakadurga

Department of ECED

Signature of External Examiner

DECLARATION

We declare that this project report titled Automatic Solar Panel Cleaning Robot submitted in partial fulfillment of the degree of Bachelor of Engineering in Electronics and Communication Engineering is a record of original work carried out by us under the supervision of Dr. B. Sarala, and has not formed the basis for the award of any other degree or diploma, in this or any other Institution or University. In keeping with the ethical practice in reporting scientific information, due acknowledgements have been made wherever the findings of others have been cited.

Student Name:                                         Roll No:                                                Signature:

M. Sri Charan Siddharth                    2451-21-735-178

Gouni Sandeep                                     2451-21-735-315

Tukkapuram Rohit Chary                  2451-21-735-317

ACKNOWLEDGEMENTS

We sincerely express my heartfelt gratitude to my supervisor Dr. B. Sarala, Professor, ECED for their invaluable guidance, unwavering support, and insightful contributions throughout this Project work. The discussions we had greatly enhanced our understanding and played a crucial role in achieving our project goals.

We also extend my gratitude to Dr. G. Kanaka Durga, Professor & Head of the Department, and our Principal, Dr. Vijaya Guntur, for their continuous support, encouragement, and for providing the necessary resources to successfully carry out this project work.

Furthermore, we would like to acknowledge the faculty and staffs of the department for their direct and indirect assistance in making this project a success.

Lastly, we are deeply grateful to my parents for their unwavering support, encouragement, and blessings throughout this journey.

M. Sri Charan Siddharth                    2451-21-735-178

Gouni Sandeep                                   2451-21-735-315

Tukkapuram Rohit Chary                   2451-21-735-317

ABSTRACT

This report presents the design and development of a fully autonomous solar panel cleaning robot aimed at enhancing the efficiency and longevity of solar energy systems. With the accumulation of dust and debris significantly reducing power output—by as much as 50% if left uncleaned for extended periods—this project addresses the critical need for a reliable maintenance solution in solar farms. The proposed robot employs advanced sensing technology and a water-assisted cleaning mechanism to operate independently, ensuring optimal performance of solar panels.

The solar panel cleaning robot is built around an Arduino microcontroller, which integrates multiple components such as infrared (IR) sensors for edge detection, a rain detection module for safety, and a cleaning mechanism featuring rotating brushes and a water pump. The robot is programmed to follow a boustrophedon cleaning path to ensure comprehensive coverage of the solar panel surface while avoiding potential hazards. Testing results demonstrated over 90% cleaning efficiency and reliable autonomous operation under various environmental conditions.

Implementation of the system involved the careful selection and integration of hardware and software tools. The Arduino platform facilitated seamless communication between sensors and motor drivers, allowing for real-time adjustments during operation. The use of cost-effective components ensured the project’s accessibility for medium to large-scale solar installations. Additionally, the potential application of this robotic solution extends beyond just cleaning; it lays the groundwork for future integration with IoT technologies for remote monitoring and scheduling, further streamlining solar panel maintenance processes.

CONTENTS

| ABSTRACT | IV |
| --- | --- |
| LIST OF FIGURES | VIII |
| LIST OF TABLES LIST OF ABBREVIATIONS | IX X |

| CHAPTER | DESCRIPTION | PAGE NO. |
| --- | --- | --- |
| 1 | Introduction | 1-3 |
|  | 1.1. Introduction | 1 |
|  | 1.2. Problem Statement | 1 |
|  | 1.3. Objectives of the Project | 1 |
|  | 1.4. Scope and Limitations | 2 |
|  | 1.4.1. Scope | 2 |
|  | 1.4.2. Limitations | 2 |
|  | 1.5. Organization of the Report | 3 |
| 2 | Discussed Literature survey of various Techniques & Comparison | 3-11 |
|  | 2.1 Literature Review | 3 |
|  | 2.1.1 Existing Methods | 3 |
|  | 2.1.2. Proposed Method | 6 |
|  | 2.2. Comparison of Various Methods | 6 |
|  | 2.3. Summary of the Analysis | 9 |
| 3 | Solar Panel Cleaning Robot | 12-41 |
|  | 3.1. Overview of the System | 12 |
|  | 3.2. Block Diagram | 13 |
|  | 3.3. Hardware Components and Specifications | 15 |
|  | 3.4. Schematic Design | 24 |
|  | 3.5. Physical Characteristics | 28 |
|  | 3.6. Software Design | 36 |
|  | 3.6.1. Pseudocode | 36 |
|  | 3.6.2. Flowchart | 40 |
|  | 3.6.3. Algorithm | 41 |
|  | 3.7. Communication Protocols Used | 41 |
|  | 3.8. Summary | 41 |
| 4 | Implementation of Solar Panel Robot | 42-47 |
|  | 4.1. Connections of Solar Panel Cleaning Robot | 42 |
|  | 4.1.1. Wiring and Connections | 42 |
|  | 4.1.2. Connection Details | 44 |
|  | 4.1.3. Circuit diagram | 45 |
|  | 4.1.4. Assembly Steps | 45 |
|  | 4.2. Software Development | 46 |
|  | 4.2.1. Programming Languages and Tools Used | 46 |
|  | 4.3. Integration of Hardware and Software | 46 |
|  | 4.4. Summary | 47 |
| 5 | Results & Discussion | 49-53 |
|  | 5.1. Test Cases and Outcomes | 49 |
|  | 5.2. Performance Analysis | 51 |
|  | 5.3. Challenges Encountered and Solutions Applied | 51 |
|  | 5.4. Overall Findings | 52 |
|  | 5.5. Results | 53 |
|  | 5.6. Summary |  |
| 6 | Conclusion & Future work | 54-58 |
|  | 6.1. Summary of the Project | 54 |
|  | 6.2. Key Findings and Contributions | 54 |
|  | 6.3. Limitations of the Current Implementation | 55 |
|  | 6.4. Possible Improvements and Future Enhancements | 55 |
|  | REFERENCES | 58 |

LIST OF FIGURES

| S. No | Name of the figure | Page No |
| --- | --- | --- |
| 3.2.2 | BLOCK DIAGRAM | 9 |
| 3.3.1 | SOLAR PANEL | 10 |
| 3.3.2 | BATTERY | 11 |
| 3.3.3 | INNER SIDE MOTOR | 13 |
| 3.3.4 | WHEEL | 16 |
| 3.3.5 | D.C WATER PUMP | 17 |
| 3.3.6 | ARDUINO BOARD | 18 |
| 3.4 | SCHEMATIC & REFERENCE DESIGN | 19 |
| 3.5.1 | L293D MOTOR DRIVER | 24 |
| 3.5.2 | L293D HARDWARE CIRCUIT | 24 |
| 3.5.3 | FRC CABLE | 26 |
| 3.5.4 | IR SENSORS | 27 |
| 4.1.3 | CIRCUIT DIAGRAM | 37 |
| 5.6 | RESULTS & OBSERVATIONS | 43 |

LIST OF TABLES

| S. No | Name of the Tables | Page No |
| --- | --- | --- |
| 3.4.1 | SPECIFICATIONS | 23 |
| 4.1.2 | TABLE OF CONNECTIONS | 41 |
| 4.2.1 | LIBRARIES USED | 43 |
| 5.1 | PERFORMANCE EVALUATION | 47 |
| 5.3 | CHALLENGES ENCOUNTERED | 48 |
| 5.3 | SOLUTIONS APPLIED | 48 |
| 6.2 | KEY FINDINGS & CONTRIBTIONS | 51 |
| 6.3.1 | CURRENT LIMITATIONS | 52 |

LIST OF ABBREVIATIONS

| Abbreviations | Definitions |  |
| --- | --- | --- |
| IOT | Internet of things |  |
| IR | Infrared |  |
| PWM | Pulse Width Modulation |  |
| PV | Photovoltaic |  |
| LED | Light Emitting Diode |  |
| TTL | Transistor-Transistor logic |  |
| ICSP USB SPI I2C UART DFU AC FRC TWI AREF EEPROM | In-Circuit Serial Programming Universal Serial Bus Serial Peripheral Interface Inter-Integrated Circuit Universal Asynchronous Receiver- Transmitter Device Firmware Upgrade Alternating Current Flat Ribbon Cable Two-Wire Interface Analog Reference Electrically Erasable PROM |  |